|  | ## CHICHESTER DISTRICT COUNCIL PLANNING GUIDANCE NOTE 5 ### PARKING STANDARDS Amended 28th September 2007 |
| --- | --- |

Maximum Car Parking Standards for main types of development (full parking standards can be viewed on WSCC & CDC websites) see explanatory notes 1-6 ##### Development Type; #### Zone 1 Chichester 500 metres of cross and bus/rail station 50% of maximum standard see also note 6; #### Zone 2 Chichester (outside of Zone 1) & Fishbourne Donnington 75% of maximum standard; #### Zone 3 Selsey, East Wittering, Midhurst, Petworth, Tangmere and East – West Bus/Rail corridor 85% of maximum standard; #### Zone 4 Remainder of District 100% Maximum standard. ##### Residential 1 bed dwellings; #### 0.50 spaces per unit; #### 1 per unit; #### 1 per unit; #### 1 space per unit. ##### 2 & 3 bed dwellings; #### 1.0 space per unit; #### 1.5 per unit; #### 2 per unit; #### 2 spaces per unit. ##### 4 bed dwellings; #### 1.5 space per unit; #### 2 per unit; #### 2.5 per unit; #### 3 spaces per unit. ##### Sheltered units; #### 1 space per 4 units; #### 1 space per 3 units; #### 1 space per 2.5 units; #### 1 space per 2 units. ##### B1 and A2 Uses; #### 1 space per 60 sqm; #### 1 space per 40 sqm; #### 1 space per 35 sqm; #### 1 space per 30 sqm. ##### B1 less than 500 sqm; #### 1 space per 50 sqm; #### 1 space per 33 sqm; #### 1 space per 25 sqm; #### 1 space per 25 sqm. ##### B2; #### 1 space per 80 sqm; #### 1 space per 53 sqm; #### 1 space per 47 sqm; #### 1 space per 40 sqm. ##### Food Retail; #### 1 space per 28 sqm; #### 1 space per 19 sqm; #### 1 space per 16 sqm; #### 1 space per 14 sqm. ##### Non Food Retail; #### 1 space per 40 sqm; #### 1 space per 27 sqm; #### 1 space per 24 sqm; #### 1 space per 20 sqm. Explanatory Notes 1. The County Parking Standards, Accessibility Zones for Chichester District and Transport Contributions Methodology were adopted by Chichester District Council on the 6th January 2004. This supersedes the parking standards and commuted car parking payments contained in the transport chapter of the adopted local plan. 2. Any urban extensions will be considered as forming part of the adjacent urban accessibility zone 3. Residential parking is different in nature to destination-type uses such as employment uses where parking restraint can be an important tool encourage people to seek alternative means of travel where alternative exist. Reducing residential parking standards outside of Chichester (the most accessible location) is likely to on-street parking problems and concreting over front gardens for extra spaces. The figures for residential parking do not therefore strictly follow the percentage reductions that are normally required for ‘destination type uses’. However the figures are maximums, and a garage counts as 1 parking space. Garages should be a minimum of 2.8 metres width internally to enable a degree of storage and cycle parking to co-exist with car parking. The PPG3 Housing guidance for a maximum of 1.5 average off-street car parking spaces will be a target. 4. Sheltered housing category applies where internal communal facilities and warden accommodation is provided. 5. All other development types, with the exception of garden centres, will be subject to the accessibility zones reductions as appropriate to the location. 6. Local Plan Policy TR2 (1) restricts parking for commercial development to operational only within the Chichester Conservation Area. This part of the policy still applies. -1- ###### Minimum Motor Cycle Parking Standard 1 space per 10 car parking spaces for commercial development Cycle Parking A safe cycle network is developing south of the downs with the hub of the network in Chichester. Elsewhere local trips by cycle are often a realistic form of transport. In all areas new development must make adequate provision for cycles to ensure this sustainable mode of transport can be easily used by occupiers of new residential and commercial property. ###### Minimum Standards 1 secure covered space per dwelling (1 bed) + 1 space per 8 dwellings for visitors 2 secure covered cycle spaces per dwelling (2 beds or more) + 1 space per 8 dwellings for visitors A1 – 1 space per 100 sqm for staff and 1 space per 100 sqm for customers A2 – 1 space per 100 sqm for staff and 1 space per 200 sqm for customers A3 – 1 space per 4 staff and 1 space per 25 sqm for customers B1 – 1 space per 150 sqm for staff and 1 cycle space per 500 sqm for visitors B2 – 1 space per 200 sqm for staff and 1 space per 500 sqm for visitors B8 – 1 space per 500 sqm for staff and 1 space per 1000 sqm for visitors Cycle Storage Design Guidance for Residential Uses Cycle storage needs to be considered at initial design and layout stage for residential developments to ensure the facility and its location is both safe and convenient and provides an adequate degree of shelter. A location overlooked by kitchens or living rooms aids safety and security. Provision needs to be secure enough for the resident to have confidence that it will provide safe undercover overnight storage. Storage within flats is not acceptable as storage space is at a premium and any cupboard will be used for general purposes. Details should be shown on the plans submitted at the time of presenting a full application. One Sheffield stand meets the cycle parking standard of 2 spaces per dwelling with 2 bedrooms or more. Stands should be marked with the flat number. Front wheel stands are unacceptable as they are prone to buckle wheels and do not provide a secure anchor point. Detached and Semi-Detached Properties Garages provided with detached or semi-detached properties will satisfy storage requirements. In all other dwelling types specific provision is required. Terraced Housing A Options for the secure and covered storage of bicycles in terraced housing 1. If rear access is provided to rear gardens, provision may be in the form of a garden shed etc. 2. Purpose built store attached to front of house/porch area 3. An internal garage Flats/Maisonettes/Bedsits Ideally individual brick built bicycle sheds (measuring 2.0 x 0.9 internally) located in a secure area. Where development is too large to accommodate individual sheds, then communal provision is permissible providing:- 1. The cycle storage area must be of solid construction to ensure the cycles cannot be seen externally. 2. No more than 6 dwellings should have to share a communal area. Larger storage buildings maybe subdivided with solid partitions internally. -2-

3. Sheffield Stands should be provided at minimum of 0.75m centres to allow cycles to be stored either side. Fixing of stands can be simplified by using a toast rack design where the bottom connecting bar can be bolted into the base. These stands provide support to the bicycle frame and allow the bicycle to be locked to an unmovable feature. Bike hooks and wall bars may also be used inside the external wall and partitions to maximise space. 4. External doors should be solid and provide with a secure lock with keys issued to each user. Campus Accommodation Sheltered bicycle sheds with Sheffield Stands are acceptable within a campus providing access is restricted to users by being within defensible space, e.g. overlooked courtyard, provision of CCTV/night porter. Reductions in the cycle parking standard may be considered acceptable at the initial development stage subject to an agreement that the provision is increased to one per unit if necessary in the light of the experience Sheltered Apartments At one end of age range are developments for persons aged 55 and above to warden aided accommodation where people are likely to be less mobile. A similar approach to campus accommodation could be made for this type of development. Office, Industry, Warehousing, Retail and Leisure Developments Cycle parking must be sheltered and secure and provided to the standard set out on page 2 of this guidance note. Only Sheffield Stands are acceptable for the reasons explained in the residential section. These should be placed at 0.75 metre minimum centres with the horizontal bar at a height of 0.75m and placed no nearer a wall than 0.55m.

Financial Contributions to Highways and Transport Improvements New development generates additional traffic on the road network and creates additional demand for better public transport, cycling and walking provision. Developments above the thresholds set out below are expected to contribute to transport improvements to meet the increased demand generated by that development. A methodology has been developed which relates contributions to ‘Total Access Demand’ (TAD). The methodology is based on total access to/from a development. An infrastructure contribution (£600) is required in respect of each occupant or employee provided with a parking space and a sustainable transport contribution (£300) is required in respect of each occupant or employee not provided with a parking space. The two elements of the contribution may be combined to mitigate the impact of movement generated by a new development. The differential between the two elements is set to encourage development in more accessible areas – more accessible locations will require lower levels of parking and hence a lower level of contribution. The development of smaller dwelling units will generate less demand for movement and will hence be required to make a lower contribution than larger units with more car parking. A full account of the methodology can be viewed on the County Council’s website http://www.westsussex.gov.uk then click on roads and transport then on WSCC transport contributions. Public consultation was carried out in accordance with government guidance and the methodology was adopted by the County Council in November 2003 and the District Council’s Executive Board on the 6th January 2004. Typical examples are given in the table below. Development; Occupancy; Parking Provision; Sustainable Access Contribution; Infrastructure Contribution; Total Contribution. 10 x 1 bed units; 13 (average 1.3 persons per unit); 1 space per unit = 10 spaces; (13-10) x £300 = £900; 10 car spaces x £600 = £6,000; £6,900. 10 x 4 bed units; 30 (average 3 persons per unit); 3 spaces per unit = 30 spaces; 0; 30 car spaces x £600 = £18,000; £18,000. B1 office – 1,000 sqm; 47.6 (1 worker per 21 sqm); 1:30 sqm = 33 spaces; (47.6 – 33) x £300 = £4,380; 33 car spaces x £600 = £19,800; £24,180. -3-

Developers may calculate the financial contribution by using the following blank table or by contacting the County Council Highways and Transport Development Control Section dealing with Chichester District. Development; Occupancy; Parking Provision; Sustainable Access Contribution; Infrastructure Contribution; Total Contribution. ##### Minimum Thresholds 1. Social housing on exception sites or on land entirely owned by a registered social landlord within an SPA will not be required to make a transport contribution. 2. 500 sqm for commercial uses in the Rural Area to facilitate rural diversification 3. 300 sqm for commercial uses in settlement policy areas 4. 5 residential units Notes 1. The sustainable access contribution and infrastructure contribution will be required in addition to contributions or improvements required to ensure that the development provides sustainable and safe access within the capacity of the transport network provide. 2. The sustainable access contribution and the infrastructure contribution will be required in addition to the costs of managing on street parking required as a result of proposed development. 3. With respect to commercial development the sustainable access contribution and infrastructure contribution will be required in addition to measures incorporated within a travel plan. 4. With respect to commercial development TAD will be based on gross floor area. 5. For residential development the methodology will be used for developments up to 100 units. Above this a full Transport Assessment will be required. 6. The methodology relates to the impact of new development – allowances will be made for movement generated by existing uses on a development site or existing parking provision provided on a site. Hence for the purposes of the contributions methodology development includes all proposals likely to result in a net increase in movement. 7. The TAD methodology provides a guide to aid transparency, consistency and equity when considering developer contributions. Exceptional circumstances will be considered on their merits. All developers have the option of providing a Transport Assessment as an alternative approach to assessing the impact of development and hence appropriate mitigation measures. 8. An assessment of wider planning objectives could result in the level of contribution being reviewed in exceptional circumstances. 9. The methodology should be considered in relation to those identified in the document titled ‘Highways and Transport Proposed Schemes to be Progressed if Developer Funding is Secured’, developed through a consultation exercise and approved by West Sussex County Council, or any other scheme which will improve access between the development and local amenities (to include housing, jobs, shops, schools, leisure and other services), approved list of schemes to be funded if developer contributions become available – contributions will be ring fenced and allocated to these schemes or others identified by the Highway Authority in partnership with the District and Borough Planning Authorities. 10. Contributions could secure improvements to facilities for the disabled, pedestrians, cyclists, public transport services or facilities, traffic management or safety enhancements. 11. Contributions will only be secured towards proposals which will influence travel patterns to/from development either on their own or as part of a package. Contributions will not be sought from development if no improvement can be identified that relates directly to movement generated by the development. 12. Contributions will be secured via a S.106 Agreement. 13. An annual report summarising the total of financial contributions received and where they have been spent will be produced and made publicly available. 14. Similar principles may be applied with respect to other use classes – however adjustments would need to be made with respect to retail development, for example, to reflect access by those other than employees or residents. 15. TAD relates directly to occupancy levels. Where local information suggests that alternative occupancy levels are more appropriate these will be considered. 16. Separate provisions apply in relation to development impacting on the trunk road network. These provisions are set out in DTLR Circular 04/2001 (Control of Development Affecting Trunk Roads and Agreements under Section 278 of the Highways Act 1980). 17. The approach will not be applied retrospectively to sites with unimplemented planning permissions. 18. The level of contributions will be reviewed annually, to include an inflation increase in accordance with relevant construction Indices. -4-